IRRIGATION SUGGESTION BOT

Technical Documentation

ESP32 Smart Irrigation Advisory Firmware

Project

IRRIGATION_SUGGESTION_BOT

Version

[ PLACEHOLDER: Version number (e.g. v1.0.0) ]

Author(s)

[ PLACEHOLDER: Author names and contact info ]

Date

[ PLACEHOLDER: Document date ]

# Table of Contents

1.  Project Overview  ..........  3

2.  Repository Structure  ..........  4

3.  Hardware Setup  ..........  5

4.  Firmware Architecture  ..........  6

5.  Module Documentation  ..........  8

5.1  config.h — Pin Definitions  ..........  8

5.2  globals — Shared State  ..........  9

5.3  sensors — Environmental Sampling  ..........  10

5.4  moisture — Soil Moisture  ..........  11

5.5  calibration — Calibration Routines  ..........  12

5.6  eto — Evapotranspiration  ..........  13

5.7  irrigation — Water Balance & Decision  ..........  15

5.8  ble_handler — BLE Interface  ..........  16

5.9  display — OLED Output  ..........  18

6.  Global Variables Reference  ..........  19

7.  Persistent Storage (NVS)  ..........  20

8.  Calibration Procedures  ..........  21

9.  Measurement Workflow  ..........  22

10.  BLE Command Reference  ..........  23

11.  Known Issues & Bugs  ..........  24

12.  Libraries & Dependencies  ..........  25

# 1. Project Overview

The Irrigation Suggestion Bot is an embedded firmware system running on the ESP32 microcontroller. It monitors environmental and soil conditions over a configurable sampling window and uses those measurements to calculate evapotranspiration (ETo) and determine whether a field requires irrigation.

The system integrates four sensor types, a Bluetooth Low Energy (BLE) interface for wireless control, a persistent calibration store in ESP32 Non-Volatile Storage (NVS), and an optional SSD1306 OLED display for local readout. It is designed for deployment in field conditions where a companion mobile app or BLE terminal sends commands and receives irrigation decisions as notifications.

## 1.1  Key Features

- 30-minute continuous environmental sensor measurement with exponential moving-average smoothing

- Soil moisture measurement at 4 configurable field points per session

- Reference Evapotranspiration (ETo) calculated via a simplified Penman-Monteith equation (FAO-56)

- Soil water balance model fusing model prediction with live sensor readings

- Full calibration suite: moisture sensor, field capacity, and irrigation application rate

- Bluetooth Low Energy interface for remote commands and status notifications

- Persistent calibration data and crop parameters stored in ESP32 NVS (survives power cycles)

- Hardware push-button for standalone operation without BLE

- OLED display for local sensor readout and results

## 1.2  System at a Glance

| Layer | Technology | Role |
| --- | --- | --- |
| Microcontroller | ESP32 (Xtensa dual-core 240 MHz) | Runs firmware, BLE stack, NVS |
| Environmental | DHT22 | Air temperature & relative humidity |
| Soil Temp | DS18B20 (OneWire) | Soil temperature measurement |
| Soil Moisture | Resistive / capacitive analog sensor | Volumetric moisture reading |
| Light | LDR / photodiode (analog ADC) | Ambient irradiance proxy for ETo |
| Communication | Bluetooth Low Energy (Nordic UART profile) | Remote control & status reporting |
| Display | SSD1306 128×64 OLED (I2C) | Local sensor readout & results |
| Storage | ESP32 NVS (Preferences library) | Persistent calibration & crop data |

# 2. Repository Structure

The project is a standard Arduino sketch folder. All source files reside in a single directory alongside the main .ino entry-point.

IRRIGATION_SUGGESTION_BOT/

IRRIGATION_SUGGESTION_BOT.ino   ← Entry point (setup, loop, ISR, state machine)

config.h                         ← Hardware pin definitions

globals.h / globals.cpp          ← Shared global variables & enumerations

sensors.h / sensors.cpp          ← DHT22 + DS18B20 sampling

moisture.h / moisture.cpp        ← Analog moisture reading & conversion

calibration.h / calibration.cpp  ← Dry / wet / FC / app-rate calibration

eto.h / eto.cpp                  ← Penman-Monteith ETo calculation

irrigation.h / irrigation.cpp    ← Soil water balance & irrigation decision

ble_handler.h / ble_handler.cpp  ← BLE server, JSON command parsing

display.h / display.cpp          ← SSD1306 OLED rendering

Each module follows a simple header / source split. Headers expose only the public API; internal state is kept in the .cpp translation unit using static or file-scope variables.

# 3. Hardware Setup

## 3.1  Pin Assignment

| Signal | ESP32 GPIO | Sensor / Device | Notes |
| --- | --- | --- | --- |
| DHT22 Data | GPIO 4 | DHT22 | 10 kΩ pull-up to 3.3 V recommended |
| DS18B20 Data | GPIO 15 | DS18B20 (OneWire) | 4.7 kΩ pull-up to 3.3 V required |
| Moisture ADC | GPIO 34 | Analog moisture sensor | Input only; no internal pull-up |
| Light ADC | GPIO 32 | LDR / photodiode | Per config.h — see conflict note below |
| Button | GPIO 25 | Tactile push button | Active LOW; PULLUP enabled in firmware |
| OLED SDA | GPIO 21 | SSD1306 OLED | I2C — 4.7 kΩ pull-up to 3.3 V |
| OLED SCL | GPIO 22 | SSD1306 OLED | I2C — 4.7 kΩ pull-up to 3.3 V |

⚠  WARNING

Pin conflict: config.h assigns MOISTURE_PIN to GPIO 34 and LIGHT_PIN to GPIO 32. However, sensors.cpp redefines LIGHT_PIN as GPIO 34, which collides with the moisture sensor. Additionally, SOIL_TEMP_PIN (GPIO 35) in config.h differs from the actual OneWire pin (GPIO 15) in sensors.cpp. Reconcile all pin definitions in config.h before deployment.

## 3.2  Wiring Diagram

[ PLACEHOLDER: Insert full wiring / schematic diagram here (Fritzing, KiCad, or hand-drawn image) ]

[ PLACEHOLDER: Insert PCB layout or breadboard photo if available ]

## 3.3  Power Requirements

[ PLACEHOLDER: Document supply voltage, typical current draw, and any power regulation components used ]

# 4. Firmware Architecture

## 4.1  Main Loop Structure

The firmware uses a two-stage main loop: a command handler that translates queued commands into state transitions, followed by a finite state machine (FSM) that drives all measurement, calibration, and decision workflows.

// IRRIGATION_SUGGESTION_BOT.ino

void loop() {

handleCommands();   // Process any pending BLE / button command

runStateMachine();  // Execute current state logic

}

Commands are deposited into the volatile global pendingCommand by either the hardware button ISR or the BLE write callback — both of which run in interrupt/task context. The main loop reads and clears the command flag safely.

// Button interrupt service routine (IRAM_ATTR — stored in fast RAM)

void IRAM_ATTR buttonISR() {

if (!measurementInProgress)

pendingCommand = CMD_START_MEASUREMENT;

else

pointConfirmed = true;

}

// Setup: attach interrupt on falling edge (button press)

pinMode(BUTTON_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(BUTTON_PIN), buttonISR, FALLING);

## 4.2  Finite State Machine

The device state is stored in the global variable currentState of type DeviceState. Every iteration of loop() calls runStateMachine(), which dispatches to the logic for the active state via a switch statement.

| State | Description | Transition To |
| --- | --- | --- |
| STATE_IDLE | Default. Awaits commands. | Any command |
| STATE_SENSOR_MEASUREMENT | Samples all sensors every 2 s for 30 minutes. | STATE_MEASUREMENT_WAIT_POINT |
| STATE_MEASUREMENT_WAIT_POINT | Collects 4 confirmed moisture point readings. | STATE_IRRIGATION_CHECK (after 4 points) |
| STATE_IRRIGATION_CHECK | Runs ETo, updates soil water balance, evaluates RAW threshold. | STATE_IDLE |
| STATE_CALIB_DRY_WAIT | Waits 3 s, records dry ADC baseline. | STATE_CALIB_WET_WAIT |
| STATE_CALIB_WET_WAIT | Waits 3 s, records wet ADC baseline. | STATE_IDLE |
| STATE_FC_WAIT | Awaits point confirmation to record field capacity. | STATE_IDLE |
| STATE_APP_RATE_BEFORE | Collects 4 pre-irrigation moisture points. | STATE_APP_RATE_AFTER |
| STATE_APP_RATE_AFTER | Collects 4 post-irrigation moisture points, computes rate. | STATE_IDLE |

## 4.3  Startup Sequence

void setup() {

// 1. Configure hardware

pinMode(BUTTON_PIN, INPUT_PULLUP);

attachInterrupt(digitalPinToInterrupt(BUTTON_PIN), buttonISR, FALLING);

Serial.begin(115200);

// 2. Initialise sensors and BLE

initSensors();

initBLE();

// 3. Load persisted calibration & crop data from NVS

prefs.begin("irrigation", false);

FC_percent       = prefs.getFloat("FC_percent",      50);

dryRaw           = prefs.getFloat("dryRaw",          0);

wetRaw           = prefs.getFloat("wetRaw",          1023);

TAW              = prefs.getFloat("TAW",             100);

RAW              = prefs.getFloat("RAW",             40);

SoilWater_mm     = prefs.getFloat("SoilWater",       TAW);

ROOT_DEPTH_MM    = prefs.getInt(  "rootDepth_mm",    ROOT_DEPTH_MM);

Kc               = prefs.getFloat("Kc",              Kc);

P_VALUE          = prefs.getFloat("pValue",          P_VALUE);

applicationRate  = prefs.getFloat("applicationRate", applicationRate);

}

# 5. Module Documentation

## 5.1  config.h — Pin Definitions

A single header providing compile-time hardware constants. All other modules should reference these macros instead of hard-coding GPIO numbers.

// config.h

#define DHTPIN        4    // DHT22 data line

#define DHTTYPE       DHT22

#define MOISTURE_PIN  34   // Analog moisture sensor

#define SOIL_TEMP_PIN 35   // ⚠ Unused — sensors.cpp uses GPIO 15

#define LIGHT_PIN     32   // ⚠ sensors.cpp redefines this as GPIO 34

#define BUTTON_PIN    25   // Active-LOW tactile button

#define NUM_POINTS    4    // Moisture readings per session

⚠  WARNING

SOIL_TEMP_PIN and LIGHT_PIN in config.h are not used by sensors.cpp, which defines its own local constants. Consolidate all pin definitions here to avoid confusion.

## 5.2  globals — Shared State

globals.h declares all shared state as extern variables; globals.cpp provides the definitions and default values. Every module that needs cross-module state includes globals.h.

// globals.h (excerpt) — enumerations

enum CommandType {

CMD_NONE,

CMD_START_MEASUREMENT,

CMD_CONFIRM_POINT,

CMD_CALIBRATE_MOISTURE,

CMD_CALIBRATE_FC,

CMD_CALIBRATE_APP_RATE,

CMD_GET_STATUS

};

enum DeviceState {

STATE_IDLE,

STATE_SENSOR_MEASUREMENT,

STATE_MEASUREMENT_WAIT_POINT,

STATE_IRRIGATION_CHECK,

STATE_CALIB_DRY_WAIT,

STATE_CALIB_WET_WAIT,

STATE_FC_WAIT,

STATE_APP_RATE_BEFORE,

STATE_APP_RATE_AFTER

};

## 5.3  sensors — Environmental Sampling

Provides initialisation and periodic sampling of the DHT22 (air temperature + humidity) and DS18B20 (soil temperature) sensors, plus an analog light level read. All four values are maintained as exponential moving averages to suppress sensor noise.

Exponential Moving Average Filter

// sensors.cpp — sampleSensors()

void sampleSensors() {

float t  = dht.readTemperature();

float h  = dht.readHumidity();

int   l  = analogRead(LIGHT_PIN);

soilSensor.requestTemperatures();

float st = soilSensor.getTempCByIndex(0);

if (!isnan(t) && !isnan(h) && st > -50) {

// Alpha = 0.1  →  time constant ≈ 19 samples (38 s at 2 s interval)

avgT     = 0.9f * avgT     + 0.1f * t;

avgRH    = 0.9f * avgRH    + 0.1f * h;

avgLight = 0.9f * avgLight + 0.1f * l;

avgSoilT = 0.9f * avgSoilT + 0.1f * st;

}

}

Bad readings (NaN from DHT or soil temperature below -50 °C) are discarded; the running averages are not updated, preserving the last valid value.

| Function | Returns | Description |
| --- | --- | --- |
| initSensors() | void | Starts DHT22 and DS18B20 libraries |
| sampleSensors() | void | Reads all sensors; updates 4 EMA values. Call every 2 s during measurement. |
| getAvgTemp() | float | EMA air temperature (°C) |
| getAvgHumidity() | float | EMA relative humidity (%) |
| getAvgLight() | float | EMA raw ADC light reading (0–4095) |
| getAvgSoilTemp() | float | EMA soil temperature (°C) |

## 5.4  moisture — Soil Moisture Measurement

Reads the analog moisture sensor, maps the raw ADC value to a 0–100 % scale using stored dry/wet calibration endpoints, and converts the percentage to an equivalent water depth in mm.

ADC Averaging & Percentage Conversion

// moisture.cpp

// Average 10 ADC readings with 50 ms gaps to reduce switching noise

int averageAnalogRead(int pin, int samples = 10) {

long sum = 0;

for (int i = 0; i < samples; i++) {

sum += analogRead(pin);

delay(50);

}

return sum / samples;

}

// Map raw ADC → 0–100 % using calibrated endpoints

float rawToMoisturePercent(int raw) {

float percent = ((float)(raw - dryRaw)) * 100.0 / (wetRaw - dryRaw);

if (percent < 0)   percent = 0;

if (percent > 100) percent = 100;

return percent;

}

// Convert moisture % → water depth: (percent / 100) * TAW

float convertToWaterMM(float moisturePercent) {

return (moisturePercent / 100.0f) * TAW;

}

| Function | Returns | Description |
| --- | --- | --- |
| averageAnalogRead(pin, samples) | int | Averages N ADC reads with 50 ms delay. Default N=10. |
| rawToMoisturePercent(raw) | float | Maps raw ADC to 0–100 % using dryRaw / wetRaw endpoints. |
| readMoisturePoint() | float | Reads MOISTURE_PIN and returns moisture percentage. |
| computeAverageMoisture(points[], count) | float | Arithmetic mean of a float array. |
| convertToWaterMM(moisturePercent) | float | Converts moisture % to equivalent soil water depth (mm). |

## 5.5  calibration — Calibration Routines

Provides four calibration functions. All results are saved to NVS immediately after measurement and are automatically reloaded on the next boot.

Moisture Sensor Dry / Wet Calibration

// calibration.cpp

void startMoistureCalibration_dry() {

dryRaw = averageAnalogRead(MOISTURE_PIN, 10);

prefs.putFloat("dryRaw", dryRaw);

}

void startMoistureCalibration_wet() {

wetRaw = averageAnalogRead(MOISTURE_PIN, 10);

prefs.putFloat("wetRaw", wetRaw);

}

Field Capacity Calibration

void startFieldCapacityCalibration() {

int moistureRaw = averageAnalogRead(MOISTURE_PIN, 10);

FC_percent = rawToMoisturePercent(moistureRaw);

// Estimate PWP as 50 % of FC (simplified Saxton–Rawls approach)

float availableFraction = (FC_percent - FC_percent * 0.5) / 100.0;

TAW         = availableFraction * ROOT_DEPTH_MM;

RAW         = TAW * P_VALUE;

SoilWater_mm = TAW;  // assume full at calibration time

prefs.putFloat("TAW",       TAW);

prefs.putFloat("RAW",       RAW);

prefs.putFloat("SoilWater", SoilWater_mm);

}

Application Rate Calibration

// Returns mm/hr based on measured moisture increase over known irrigation time

float calibrateApplicationRate(float before_mm,

float after_mm,

float irrigationTime_minutes) {

if (irrigationTime_minutes <= 0) return 0;

float waterAdded  = after_mm - before_mm;

if (waterAdded <= 0) return 0;

float time_hours  = irrigationTime_minutes / 60.0;

return waterAdded / time_hours;   // mm / hr

}

## 5.6  eto — Evapotranspiration Calculation

Implements a simplified Penman-Monteith reference evapotranspiration (ETo) formula, adapted from the FAO-56 guidelines for use on resource-constrained embedded hardware.

ETo Formula Constants

// eto.cpp — site and model constants

#define ALTITUDE  100.0f   // metres above sea level

#define RAD_A     0.0098f  // linear slope:     Rn = RAD_A * lightADC + RAD_B

#define RAD_B    -0.85f    // linear intercept  (requires site calibration!)

// ⚠  KC is a local macro set to 1.05 — it shadows the global Kc crop-coefficient.

//    This is a bug; calculateETc ignores the user-supplied Kc.

#define KC  1.05f

ETo Calculation

float calculateETo(float avgT, float avgRH, float avgLight, float avgSoilT) {

// Saturation vapour pressure (kPa)

float es    = 0.6108f * exp((17.27f * avgT) / (avgT + 237.3f));

// Actual vapour pressure (kPa)

float ea    = (avgRH / 100.0f) * es;

// Slope of saturation vapour pressure curve (kPa / °C)

float delta = (4098.0f * es) / ((avgT + 237.3f) * (avgT + 237.3f));

// Atmospheric pressure from altitude (kPa)

float P     = 101.3f * pow((293.0f - 0.0065f * ALTITUDE) / 293.0f, 5.26f);

// Psychrometric constant (kPa / °C)

float gamma = 0.000665f * P;

// Net radiation proxy from ADC light reading (MJ m⁻² day⁻¹ equivalent)

float Rn    = RAD_A * avgLight + RAD_B;

// Soil heat flux proxy

float G     = 0.1f * (avgSoilT - avgT);

// Simplified Penman-Monteith (wind term omitted — set to constant 0.06)

float ETo   = (0.408f * delta * (Rn - G)) + (0.06f * (es - ea));

ETo *= 10.0f;   // unit scaling

return ETo;

}

float calculateETc(float eto, float Kc) {

return eto * KC;   // ⚠ uses local macro KC=1.05, ignores parameter Kc

}

⚠  WARNING

The RAD_A / RAD_B coefficients convert a raw ADC reading to net radiation. These are empirical estimates that must be validated against a calibrated pyranometer at the deployment site before production use.

ℹ  NOTE

The wind speed term in the Penman-Monteith equation is hard-coded to a constant (0.06). For accurate results in windy climates, add an anemometer and extend calculateETo() with a real wind-speed input.

## 5.7  irrigation — Water Balance & Irrigation Decision

Maintains a running estimate of soil water content (SoilWater_mm) by subtracting daily ETc and fusing the model estimate with the fresh sensor reading using a weighted blend.

// irrigation.cpp

void updateSoilWater(float measuredWater_mm) {

// 1. Subtract crop water use

float etc     = calculateETc(ETO, Kc);

SoilWater_mm -= etc;

// 2. Fuse model estimate with sensor measurement (70 % model, 30 % sensor)

SoilWater_mm  = 0.7f * SoilWater_mm + 0.3f * measuredWater_mm;

// 3. Clamp to physically valid range [0, TAW]

SoilWater_mm  = constrain(SoilWater_mm, 0, TAW);

}

// Irrigation is needed when soil water falls to or below the

// Readily Available Water (RAW) depletion threshold.

bool irrigationNeeded() {

return SoilWater_mm <= RAW;

}

The 70/30 blending ratio trades off model drift against sensor noise. Tune this coefficient once field-deployment data is available.

## 5.8  ble_handler — BLE Interface

Implements a Nordic UART Service (NUS)-style BLE server. The device advertises as IrrigationDevice. A single characteristic supports read, write, and notify. The BLE write callback parses incoming JSON commands and queues the appropriate CommandType into pendingCommand.

BLE Server Initialisation

// ble_handler.cpp

#define SERVICE_UUID        "6E400001-B5A3-F393-E0A9-E50E24DCCA9E"

#define CHARACTERISTIC_UUID "6E400002-B5A3-F393-E0A9-E50E24DCCA9E"

void initBLE() {

BLEDevice::init("IrrigationDevice");

BLEServer    *pServer  = BLEDevice::createServer();

pServer->setCallbacks(new MyServerCallbacks());

BLEService   *service  = pServer->createService(SERVICE_UUID);

pCharacteristic = service->createCharacteristic(

CHARACTERISTIC_UUID,

BLECharacteristic::PROPERTY_READ   |

BLECharacteristic::PROPERTY_WRITE  |

BLECharacteristic::PROPERTY_NOTIFY

);

pCharacteristic->setCallbacks(new MyCallbacks());

pCharacteristic->addDescriptor(new BLE2902());

service->start();

BLEDevice::getAdvertising()->start();

}

JSON Command Parsing

// MyCallbacks::onWrite() — called when client writes to the characteristic

void onWrite(BLECharacteristic *pChar) {

std::string rx = pChar->getValue();

if (rx.empty()) return;

StaticJsonDocument<256> doc;

if (deserializeJson(doc, rx)) { sendBTStatus("JSON Error"); return; }

String cmd = doc["command"] | "";

if      (cmd == "measure")              pendingCommand = CMD_START_MEASUREMENT;

else if (cmd == "point")                pendingCommand = CMD_CONFIRM_POINT;

else if (cmd == "calibrate_moisture")   pendingCommand = CMD_CALIBRATE_MOISTURE;

else if (cmd == "calibrate_fc")         pendingCommand = CMD_CALIBRATE_FC;

else if (cmd == "calibrate_app_rate")   pendingCommand = CMD_CALIBRATE_APP_RATE;

else if (cmd == "crop_params") {

TAW = doc["TAW"] | TAW;

RAW = doc["RAW"] | RAW;

prefs.putFloat("TAW", TAW);

prefs.putFloat("RAW", RAW);

sendBTStatus("Crop Parameters Saved");

}

}

BLE Command Reference Table

| JSON Command | Effect | Extra JSON Fields |
| --- | --- | --- |
| {"command":"measure"} | Starts 30-min measurement session | — |
| {"command":"point"} | Confirms current moisture sampling point | — |
| {"command":"calibrate_moisture"} | Begins dry → wet sensor calibration | — |
| {"command":"calibrate_fc"} | Begins field capacity calibration | — |
| {"command":"calibrate_app_rate"} | Begins application rate calibration | — |
| {"command":"status"} | Reserved — currently no-op | — |
| {"command":"crop_params",...} | Sets TAW and RAW; saves to NVS | "TAW":<float>, "RAW":<float> |

Status Notifications

sendBTStatus(msg) pushes a plain-text UTF-8 string as a BLE notification. Representative messages:

| Notification String | Meaning |
| --- | --- |
| Starting 30min measurement | Measurement session has begun |
| Point Recorded | One of the 4 moisture points has been recorded |
| All Points Measured | All 4 points collected; averaging complete |
| Moisture Calibration Done | Dry and wet calibration saved to NVS |
| Field Capacity Saved | FC calibration result saved to NVS |
| Crop Parameters Saved | TAW / RAW updated and persisted |
| Calibration Completed | Application rate calibration finished |
| JSON Error | Received payload was not valid JSON |

## 5.9  display — OLED Output

Drives a 128×64 SSD1306 OLED connected via I2C (SDA GPIO 21, SCL GPIO 22, address 0x3C). Provides two render modes: a live progress view during measurement and a results summary view.

// display.cpp

void initDisplay() {

Wire.begin(21, 22);

display.begin(SSD1306_SWITCHCAPVCC, 0x3C);

display.clearDisplay();

display.setTextSize(1);

display.setTextColor(WHITE);

}

// Shows live sensor readings and a graphical progress bar

void showProgress(float percent, float t, float soilT, float rh) {

display.clearDisplay();

display.printf("Air:  %.1f C\n",       t);

display.printf("Soil: %.1f C\n",       soilT);

display.printf("RH:   %.1f %%\n",      rh);

display.printf("Progress: %.0f %%\n",  percent);

drawBar(percent);   // fills proportional rect inside 110-px border

display.display();

}

// Shows ETo, ETc and equivalent water depth after a completed session

void showResult(float eto, float etc) {

display.clearDisplay();

display.printf("ETo:   %.2f mm\n", eto);

display.printf("ETc:   %.2f mm\n", etc);

display.printf("Water: %.2f L/m2\n", etc);

display.display();

}

⚠  WARNING

initDisplay() is defined but NOT called in setup(). Add initDisplay() to the setup() function in IRRIGATION_SUGGESTION_BOT.ino to enable the OLED.

# 6. Global Variables Reference

All shared state is declared extern in globals.h and defined in globals.cpp. Modules include globals.h to access or modify shared variables.

| Variable | Type | Default | Description |
| --- | --- | --- | --- |
| moisturePoints[4] | float[] | {0} | 4 moisture readings per session (%) |
| pointIndex | int | 0 | Write cursor into moisturePoints |
| avgMoisture | float | 0 | Mean of the 4 collected moisture points |
| prefs | Preferences | — | ESP32 NVS handle (namespace: irrigation) |
| dryRaw | float | 0 | ADC reading at fully dry soil |
| wetRaw | float | 1023 | ADC reading at fully saturated soil |
| SoilWater_mm | float | 50 | Running soil water content estimate (mm) |
| measured_water | float | 0 | Sensor-derived soil water depth (mm) |
| TAW | float | 100 | Total Available Water in root zone (mm) |
| RAW | float | 40 | Readily Available Water — irrigation trigger (mm) |
| FC_percent | float | 50 | Field capacity moisture (%) |
| PWP_percent | float | 0 | Permanent wilting point (%) |
| ROOT_DEPTH_MM | int | 300 | Root zone depth (mm) |
| Kc | float | 1.0 | FAO-56 crop coefficient |
| P_VALUE | float | 0.4 | Depletion fraction p (0–1) |
| ETO | float | — | Most recent ETo result (mm/day) |
| applicationRate | float | 5 | Irrigation application rate (mm/hr) |
| irrigationTime | float | 60 | Irrigation run time used in rate calibration (min) |
| appRateBefore/After | float | 0 | Pre / post moisture readings for rate calibration |
| measurementInProgress | bool | false | True while 30-min sensing session is active |
| pointConfirmed | bool | false | Set true when user confirms a sampling point |
| pendingCommand | volatile CommandType | CMD_NONE | Command queued from ISR or BLE for main loop |
| currentState | DeviceState | IDLE | Active FSM state |
| stateStartTime | unsigned long | 0 | Timestamp of last state entry (ms) |
| lastSensorSample | unsigned long | 0 | Timestamp of last sensor sample (ms) |
| sensorStartTime | unsigned long | 0 | Timestamp of current measurement session start (ms) |

# 7. Persistent Storage (NVS)

The ESP32 Preferences library is used to persist calibration results and crop parameters across power cycles. All keys live under the namespace "irrigation".

| NVS Key | Type | Default | Written By |
| --- | --- | --- | --- |
| FC_percent | float | 50 | startFieldCapacityCalibration() |
| dryRaw | float | 0 | startMoistureCalibration_dry() |
| wetRaw | float | 1023 | startMoistureCalibration_wet() |
| TAW | float | 100 | startFieldCapacityCalibration() / BLE crop_params |
| RAW | float | 40 | startFieldCapacityCalibration() / BLE crop_params |
| SoilWater | float | TAW | startFieldCapacityCalibration() |
| rootDepth_mm | int | 300 | Not yet exposed via BLE — set in source |
| Kc | float | 1.0 | Not yet exposed via BLE — set in source |
| pValue | float | 0.4 | Not yet exposed via BLE — set in source |
| applicationRate | float | 5 | calibrateApplicationRate() |

ℹ  NOTE

rootDepth_mm, Kc, and pValue can only be changed by modifying globals.cpp and re-flashing. Exposing these via the BLE crop_params command is recommended for field flexibility.

# 8. Calibration Procedures

## 8.1  Moisture Sensor Calibration

Establishes the dry and wet ADC endpoints for the rawToMoisturePercent() mapping.

- Send {"command": "calibrate_moisture"} via BLE (or press the button if mapped).

- Insert the sensor into completely dry soil or expose it to air. The device waits 3 seconds, then records dryRaw.

- Insert the sensor into fully saturated (dripping-wet) soil. The device waits 3 seconds, then records wetRaw.

- Receive 'Moisture Calibration Done' notification — values saved to NVS.

## 8.2  Field Capacity Calibration

Records the soil moisture at field capacity and derives TAW, RAW, and initial SoilWater_mm.

- Saturate the soil and allow it to drain freely for 24–48 hours until drainage has stopped (field capacity condition).

- Insert the sensor and send {"command": "calibrate_fc"}.

- Confirm with {"command": "point"} (or button press).

- Receive 'Field Capacity Saved' notification.

## 8.3  Application Rate Calibration

Measures how many mm of water the irrigation system applies per hour.

- Send {"command": "calibrate_app_rate"} to enter before-irrigation mode.

- Confirm 4 soil moisture points (send "point" four times with sensor at different locations).

- Run the irrigation system for a known, fixed duration. The irrigationTime variable (default 60 min) must match this duration.

- After irrigation, confirm 4 post-irrigation moisture points.

- The system computes and saves applicationRate (mm/hr) automatically.

# 9. Measurement Workflow

A complete irrigation-check cycle from trigger to decision proceeds as follows:

- Trigger the measurement via BLE {"command": "measure"} or by pressing the hardware button.

- The device enters STATE_SENSOR_MEASUREMENT. Every 2 seconds it calls sampleSensors(), updating EMA values for air temperature, humidity, light, and soil temperature. This continues for 30 minutes (1 800 000 ms).

- After 30 minutes the device transitions to STATE_MEASUREMENT_WAIT_POINT and sends a 'Confirm Moisture Points' BLE notification.

- The user physically moves the moisture probe to 4 representative locations in the field and sends {"command": "point"} at each position. Each call records readMoisturePoint() into moisturePoints[].

- After the 4th point, the average moisture is calculated and convertToWaterMM() converts it to measured_water (mm).

- The device enters STATE_IRRIGATION_CHECK: calculateETo() runs on the 30-min averages, updateSoilWater() fuses the ETo model with measured_water, and irrigationNeeded() evaluates whether SoilWater_mm <= RAW.

- The result (irrigation needed or not) is communicated via BLE notification and displayed on the OLED. The device returns to STATE_IDLE.

ℹ  NOTE

The 30-minute measurement window captures a representative snapshot of midday conditions. For best results, trigger a measurement at the peak demand period of the day (typically 10:00–14:00).

# 10. BLE Command Reference

## 10.1  Service Information

| Property | Value |
| --- | --- |
| Device Name | IrrigationDevice |
| Service UUID | 6E400001-B5A3-F393-E0A9-E50E24DCCA9E |
| Characteristic UUID | 6E400002-B5A3-F393-E0A9-E50E24DCCA9E |
| Characteristic Props | READ ¦ WRITE ¦ NOTIFY |
| Payload Format | UTF-8 JSON (write direction); UTF-8 plain text (notify direction) |

## 10.2  All Commands

| Command | JSON Payload | State Required | Response Notification |
| --- | --- | --- | --- |
| Start measurement | {"command":"measure"} | STATE_IDLE | Starting 30min measurement |
| Confirm point | {"command":"point"} | WAIT_POINT or CALIB states | Point Recorded / Point Confirmed |
| Calibrate moisture | {"command":"calibrate_moisture"} | STATE_IDLE | Moisture Calibration Start → Done |
| Calibrate FC | {"command":"calibrate_fc"} | STATE_IDLE | FC Calibration Start → Field Capacity Saved |
| Calibrate app rate | {"command":"calibrate_app_rate"} | STATE_IDLE | App Rate Calibration Start → Calibration Completed |
| Set crop parameters | {"command":"crop_params","TAW":90,"RAW":36} | Any | Crop Parameters Saved |
| Status (reserved) | {"command":"status"} | Any | (none — no-op) |

# 11. Known Issues & Bugs

| ID | Severity | Description | Fix |
| --- | --- | --- | --- |
| BUG-01 | High | Pin conflict: MOISTURE_PIN and LIGHT_PIN both resolve to GPIO 34 after sensors.cpp redefines LIGHT_PIN. | Assign light sensor to a different ADC GPIO (e.g. GPIO 36) and update sensors.cpp. |
| BUG-02 | High | SOIL_TEMP_PIN in config.h (GPIO 35) is never used. sensors.cpp hard-codes SOIL_PIN = 15. | Remove SOIL_TEMP_PIN from config.h or set it to 15 and reference it in sensors.cpp. |
| BUG-03 | High | #define KC 1.05 in eto.cpp shadows the global float Kc. calculateETc() always uses 1.05 regardless of user-set crop coefficient. | Remove #define KC and use the Kc parameter passed into calculateETc(). |
| BUG-04 | Medium | initDisplay() is never called in setup(), so the OLED remains uninitialised. | Add initDisplay() to setup() in IRRIGATION_SUGGESTION_BOT.ino. |
| BUG-05 | Medium | averageAnalogRead() is defined both in sensors.cpp (static) and moisture.cpp, causing a duplicate-symbol risk. | Remove the definition from moisture.cpp and expose it via sensors.h. |
| BUG-06 | Medium | CMD_GET_STATUS ('status' command) handler is commented out — the command is a no-op. | Implement a status response that sends SoilWater_mm, ETO, and irrigation decision over BLE. |
| BUG-07 | Low | irrigationTime (default 60 min) cannot be changed via BLE. Application rate calibration silently uses the wrong duration if irrigation runs for a different time. | Add irrigationTime to the crop_params BLE command. |
| BUG-08 | Low | RAD_A / RAD_B coefficients are untested empirical estimates. ETo results will be inaccurate without site-specific calibration. | Calibrate against a reference pyranometer and update constants. |

# 12. Libraries & Dependencies

| Library | Version | Purpose | Install via |
| --- | --- | --- | --- |
| DHT sensor library | ≥1.4 | DHT22 temperature & humidity | Arduino Library Manager (Adafruit) |
| OneWire | ≥2.3 | 1-Wire protocol for DS18B20 | Arduino Library Manager |
| DallasTemperature | ≥3.9 | DS18B20 temperature abstraction | Arduino Library Manager |
| ESP32 BLE Arduino | built-in | BLE server, characteristic, GATT | ESP32 Arduino core (built-in) |
| ArduinoJson | ≥6.21 | JSON parsing of BLE command payloads | Arduino Library Manager (Benoit Blanchon) |
| Adafruit GFX Library | ≥1.11 | Graphics primitives for OLED | Arduino Library Manager (Adafruit) |
| Adafruit SSD1306 | ≥2.5 | SSD1306 I2C OLED driver | Arduino Library Manager (Adafruit) |
| Preferences | built-in | ESP32 NVS key-value storage | ESP32 Arduino core (built-in) |
| Wire | built-in | I2C bus for OLED | Arduino core (built-in) |
| math.h | stdlib | exp() / pow() for ETo formula | C standard library |

[ PLACEHOLDER: List exact tested library versions after validation on target hardware ]