Smart Portable Irrigation Advisor

Project Documentation

1️⃣ Project Overview

Modern farmers often irrigate based on visual estimation, leading to overwatering, underwatering, or wasted water resources. This device is a portable, multi-point, offline IoT irrigation advisor that calculates field-specific water requirements using soil and environmental data.

Key features:

- Multi-point soil moisture measurement (3–4 points per session)

- Environmental sensing (temperature, humidity, light)

- Soil water balance calculation (TAW, RAW, ETc)

- Crop-specific configuration via BLE mobile app

- Offline working, persistent storage of calibration and crop data

- Irrigation suggestion notifications via BLE

2️⃣ Objectives & Problem Statement

Problem: Farmers lack precise water requirement data; existing solutions are either expensive, require continuous connectivity, or focus on farm-wide irrigation without field-level accuracy.

Objective:

Create a portable, offline device that can:

- Measure soil and environmental parameters at multiple points.

- Calculate available water and irrigation requirement.

- Provide accurate irrigation advice.

- Be configurable for different crops and field conditions.

- Store historical measurements and crop parameters.

3️⃣ Hardware Components

| Component | Purpose |
| --- | --- |
| ESP32 | Microcontroller, BLE, processing, offline calculations |
| DHT22 | Ambient temperature & humidity sensor |
| DS18B20 | Soil temperature sensor |
| Capacitive Soil Moisture Sensor (v2.1) | Soil moisture measurement |
| Ambient Light Sensor | Solar radiation approximation for ETo calculation |
| Push Button | Manual trigger / calibration confirmation |
| NVS (ESP32 Internal Storage) | Stores calibration, crop config, and soil water |

4️⃣ Sensor Calibration & Measurement Techniques

Soil Moisture Calibration:

- Dry Soil Calibration: Insert sensor into completely dry soil → record raw ADC value (dryRaw).

- Wet Soil Calibration: Insert sensor into fully saturated soil → record raw ADC value (wetRaw).

- Convert ADC reading to percent moisture: Moisture Percent = ((Raw - DryRaw)/(WetRaw - DryRaw)) * 100.

Field Capacity (FC) Calibration:

- Insert sensor into irrigated field after drainage.

- Measure raw soil moisture → convert to percent (FC_percent).

- Permanent Wilting Point (PWP) = 50% of FC_percent.

- TAW (Total Available Water) =

(FC_percent - PWP_percent)/100 * RootDepth_mm.

- RAW (Readily Available Water) = TAW * P.

5️⃣ Firmware Logic & Flow Diagram

Stepwise Firmware Flow:

- Trigger Session → BLE command or Button press

- Environmental Measurement → Measure temperature, humidity, light once

- Multi-Point Soil Moisture:

- Prompt farmer for each point → wait for button press

- Measure moisture → store in array

- Average Soil Moisture → calculate measured water in mm

- Soil Water Balance → update SoilWater_mm with ETc & measured water

- Irrigation Suggestion → check if SoilWater_mm <= RAW → send BLE message

- End Session → store values in NVS

Flow Diagram:

| Parameter | Formula / Explanation |
| --- | --- |
| Moisture Percent | ((Raw - DryRaw)/(WetRaw - DryRaw)) * 100 |
| Field Capacity (FC) | Measured moisture at drained field |
| PWP | 0.5 * FC |
| TAW (Total Available Water) | (FC - PWP)/100 * RootDepth |
| RAW (Readily Available Water) | TAW * P |
| ETo (Simplified Hargreaves) | 0.5 * (AirTemp + 17.8) * LightFactor |
| ETc | ETo * Kc |
| SoilWater_mm | SoilWater_mm = 0.7*SoilWater_mm + 0.3*MeasuredWater_mm |

7️⃣ Code Explanation (Key Sections)

- BLE Setup: Handles asynchronous commands via callbacks.

- Button ISR: Triggers session start and point confirmation.

- Multi-Point Measurement Loop: Waits for button press, reads soil moisture, stores in array, averages values.

- Soil Water Balance: Computes ETc, updates SoilWater_mm, checks irrigation.

- Calibration: Moisture & Field Capacity, persistent storage.

8️⃣ BLE & User Interaction

| Action | Trigger | Response |
| --- | --- | --- |
| Start Measurement | BLE / Button | Device measures env + multi-point soil moisture |
| Configure Crop | BLE JSON | Device measures env + multi-point soil moisture |
| Calibrate Moisture | BLE | Guide through dry/wet calibration |
| Calibrate FC | BLE | Guide through field capacity calibration |
| Get Status | BLE | Send soil & environmental data |

9️⃣ Operational Workflow (Farmer Use)

- Turn on device → BLE ready

- Start measurement via app or button

- Insert sensor at Point 1 → press button → repeat for all points

- Device averages soil moisture, computes soil water balance

- Device sends irrigation suggestion via BLE

- Data stored for history & crop management

1️⃣0️⃣ Future Improvements / Adaptive Learning

- Auto-TAW adaptation based on repeated measurements

- Dynamic Kc adjustment based on crop growth stage

- Offline logging for trend analysis

- Battery optimization: sleep mode between measurements

- Optional GSM module for cloud alerts